Western Blotting Protocol ­­– SDS-PAGE

NOTES:

--Do not reuse running buffer--

--10% polyacrylamide gel: 10-150 kDa--

--10-20% gradiant gel: 6 to 150 kDa--

--4-12% tris-glycine gel (30 to 200 kDa) 10 well, 30 uL volume--

--8.6 x 6.7 x 0.1 cm mini-PROTEAN TGX Pre-cast gel

1) Quantify protein samples, must have ≥ 5 ug  (≥ 0.25 ug/uL)

2) Prepare buffers:

Sample loading buffer (125 uL)

6 uL βME

119 uL Laemmli buffer (contains bromophenol blue)

Running buffer (750 mL)

675 mL water

75 mL 10x TGS w/SDS (Tris-glycine pH 8.3, w/ 0.1% SDS)

750mg SDS (if needed)

3) Prepare protein samples

-Mix equal amount of protein (5-25ug) with loading buffer / water

-Max of 28uL for 30uL well gels

I.E.  5-18 uL sample/water + 5-12 uL loading buffer

-Spin, then heat at 95°C 5', then spin

3) Electrophoresis

-Rinse wells

-Pre-run empty gel 5'

-Load samples

-Load 5-10uL Chameleon duo pre-stained ladder in center well (ready to use)

-Run gel 5-10V/cm first 10' (100V)

-Run gel 10-20V/cm 25' (200V)

-Watch dye front, keep temperature stable @ 15°C

4) Open cassette and remove gel by floatation

Western Blotting Protocol – TRANSFER

NOTES:

--PVDF membrane is hydrophobic, binding capacity is 155 ug/cm2 (Insulin)--

1) Prepare buffers:

Transfer buffer (1,000 mL CHILLED)

100 mL 10x Tris-glycine buffer, pH 8.3

800 mL Water

100 mL Methanol (10 % final)

2) Equilibrate gel 15-30'

-Large volume transfer buffer allows for SDS stripping

-Preserves dimensional equilibrium

3) Prepare PVDF membrane

-Cut to size 8.6 x 6.7 cm

-Wet PVDF in ≥ 50% metOH (turns transparent)

-Submerse in water, do not allow to float 2'

-Rinse in transfer buffer 5'

4) Setup transfer sandwhich (all in transfer buffer):

(black) ­– cathode

soaked sponge ­– push out airbubbles (clean in 100% methanol)

soaked filter paper

Gel

Membrane

soaked filter paper

soaked sponge – push out airbubbles (clean in 100% methanol)

(clear/red) + anode

5) Transfer 60-120' in ice bath

-6-8 V/cm interelectrode distance (32V)

6) Dry membrane

-Rinse in water

-Air dry at room temp 2 hours

-Store up to 2 weeks at 4°C

Western Blotting Protocol – STAINING

1) Wetting

-100% MetOH 1'

-Rinse in water 2'

-Rinse in TBS (NO tween) 5'

2) Block 1 hour (shaking)

-100 % LiCor block (or 0.2 to 5 % BSA in TBS)

-0.4 mL/cm2 (full 58 cm2 gel = 23 mL)

-NO tween

3) Prepare solutions:

TBST Wash Buffer (500 mL, 0.1 % tween)

50 mL 10x TBS buffer

450 mL Water

0.5 mL Tween 20

TBST Blocking Buffer (500 mL, 0.1 % tween)

500 mL LiCor TBS blocking buffer

0.5 mL Tween 20

TBST Blocking Buffer w/SDS (100 mL, 0.01 % SDS)

100 mL TBST Blocking Buffer

10 mg SDS

3) Incubate primary 1-4 hours @ room temp shaking (overnight 4C)

-1:200 to 1:5,000 in TBST Blocking Buffer (1:1,000 ERK)

- ≥ 0.1 mL solution/cm2 membrane (full 58 cm2 gel = 5.8 mL)

4) Wash 3x in TBST Wash Buffer

5) Incubate secondary 1 hour @ room temp shaking

-1:5,000 to 1:25,000 (1:20,000 starting)

-TBST Blocking Buffer w/SDS

6) Wash 3x in TBST Wash Buffer

-Final wash in TBS (NO tween)

7) Scan within 1 week if wet, 2-3 years if dry

-Cannot strip once dried

Western Blotting Protocol – ANTIBODIES

Primary

Rabbit-Anti-NefH                           	200 kDa      	1:1,000       	IgG  	(Chp+) Sigma

Rabbit-Anti-Phospho-IGF-I          	95 kDa        	1:1,000       	IgG  	(Ch?)   CST

*Mouse-Anti-MAP2                       	70 kDa        	1:1,000       	IgG1	(Ch+)  Sigma

*Rabbit-Anti-PhosphoAKT           	60 kDa        	1:1,000       	IgG  	(Ch+)  Sigma

*Mouse-Anti-beta3 Tub                	50 kDa        	1:1,000       	IgG2a  (Ch?)	Therm

Rabbit-Anti-beta3 Tub                  	50 kDa        	1:1,000       	IgG  	(Ch+)  Therm

*Rabbit-Anti-Phospho-ERK1/2   	42 & 44 kDa   1:1,000                   	(Chp+)  CST

*Mouse-Anti-ERK1/2                    	42 & 44 kDa   1:1,000       	IgG1	(Ch+) Millip

*Mouse-Anti-COX IV                      	17 kDa        	1:1,000       	IgG1	(Ch-)   CST

Secondary

Goat-Anti-Mouse IRDye 680RD   	For abundant proteins

Goat-Anti-Rabbit IRDye 800CW  	For low abundance proteins